Solar Car Challenge

Rollbar Worksheet

Warning: Information will be verified at scrutineering.  Cars weighing more then the listed value or not matching the given information will be disqualified.

- Mass of the vehicle (m)							____________ Kg

- Roll Bar Height (f)								____________ m

- Roll Bar Width	(L)								____________ m

- Roll Bar Outside Diameter (D)							____________ m

- Roll Bar Wall Thickness (t)							____________ m

- Material Type									____________

- Yield Strength									____________MPa

Step One

Calculate 5G Load

- Load (P)				P=5*9.8 ms2*m			____________ N

Step Two

Calculate Moment of Inertia for tube

- Roll Bar inner diameter (d)		 d=D-(2*t)			_____________ m

- Moment of Inertia (I)			I=D4-d4*π64				______________ m4

Step Three

Calculate Reaction Forces and Moments

- Vertical Reaction Force (Ra)								___________N

- Horizontal Reaction Force (Ha)							___________N

- Peak Moment (MB)									___________N*m

Step Four

Calculate Resulting stress

- Bending Stress (σb)			σb=MB*D2I					__________ MPa

- Sheer Stress (σs)			σs=HaπD22-d22				__________ MPa

Step Five

Determine Suitability

Bending Stress (σb)[Line 14]  &                                            Sheer Stress σs[Line 15] <                                            Yield Strength [Line 7]

I certify that the information presented is accurate and I believe the material selection meets the requirements of the rules and is appropriate for driver safety.

_____________________________				______________________________

Team Captain							Sponsor